Team Name/Members: [전남규(2020271319), 전예규(2025270607), 조승찬(2022270620), 조석호(2021270668)]

Project Title: 자율주행 고도화를 위한 지연 시간 감수형 네트워크(Latency-Critical Networks)의 기술적 분석 및 대응

1. Motivation

1.1. 산업 동향: 5G-Advanced 및 6G 기반의 초신뢰 저지연 통신(URLLC) 진화

차세대 통신 기술의 비약적인 발전은 자율주행 산업의 패러다임을 근본적으로 변화시키고 있다. 현재 네트워킹 업계는 단순한 데이터 전송 효율성을 넘어, 엄격한 시간 제약 조건을 만족해야 하는 지연 시간 감수형 네트워크(Latency-Critical Networks, LCN)로의 전환을 맞이하고 있다. 특히 3GPP Release 16과 17을 통해 정립된 초신뢰 저지연 통신(Ultra-Reliable Low-Latency Communications, URLLC) 기술은 자율주행 차량이 요구하는 1ms 이하의 지연 시간과 99.999% 이상의 신뢰성을 보장하는 핵심 동력이다. 이러한 흐름은 5G-Advanced(Release 18)를 거쳐 6G로 이어지며, 네트워크 자체가 인지(Sensing)와 통신(Communication)을 통합하는 방향으로 진화하고 있다.

글로벌 완성차 제조사들과 통신 장비 기업들은 이제 차량을 하나의 '움직이는 데이터 센터'로 정의하고 있다. 현대자동차와 현대모비스는 5G와 V2X(Vehicle-to-Everything) 통합 솔루션을 통해 레벨 3 이상의 자율주행 기술을 상용화하고 있으며, 이는 고속도로 주행 파일럿(HDP)과 같은 고도의 자율주행 기능을 가능하게 한다. 이러한 시스템은 차량 내부의 수많은 센서(라이다, 카메라, 레이더)가 생성하는 수 기가비트(Gbps) 규모의 데이터를 실시간으로 처리하고, 주변 인프라와 즉각적으로 공유해야 하므로 결정론적 네트워킹(Deterministic Networking) 기술의 도입이 필수적이다.

1.2. 실질적 관련성: 자율주행 안전성 확보와 에지 컴퓨팅의 결합

자율주행 환경에서 네트워크의 성능은 편의성을 넘어 승객의 생명과 직결되는 안전의 핵심 요소이다. 기존의 베스트 에포트(Best-effort) 방식 네트워크는 혼잡 상황에서 패킷 지연이나 손실을 피할 수 없으며, 이는 자율주행 차량의 제어 루프를 붕괴시킬 위험이 있다. 현실 세계의 자율주행 시나리오에서는 시속 100km 이상의 고속 주행 시 불과 수 밀리초의 지연만으로도 수 미터의 제동 거리 차이가 발생하며, 이는 사고의 유무를 결정짓는 결정적 변수가 된다.

이러한 문제를 해결하기 위해 멀티액세스 에지 컴퓨팅(Multi-access Edge Computing, MEC)이 자율주행 인프라의 필수 구성 요소로 자리 잡았다. MEC는 데이터를 중앙 클라우드 서버까지 보내지 않고 네트워크의 말단(Edge)에서 즉각 처리함으로써 왕복 지연 시간(RTT)을 획기적으로 단축한다. AWS, Google, Cisco와 같은 글로벌 IT 리더들은 Verizon 등 통신사와의 협력을 통해 사설 5G와 MEC를 결합한 아키텍처를 구축하고 있으며, 이를 통해 스마트 항만, 자동 운송 로봇(AGV), 지능형 교통 시스템(ITS) 등 실제 산업 현장에서의 자율주행 실용성을 증명하고 있다.

1.3. 직무 역량 가치: 차세대 네트워크 전문가를 위한 실무 지식 습득

본 프로젝트를 통해 지연 시간 감수형 네트워크의 설계 원리와 프로토콜 메커니즘을 마스터하는 것은 현대 네트워킹 및 IT 산업에서 요구하는 최상위 기술 역량을 확보하는 과정이다. 단순히 교과서적인 네트워크 기초를 넘어 IEEE 802.1 TSN(Time-Sensitive Networking), IETF DetNet(Deterministic Networking), 3GPP URLLC와 같은 산업 표준을 깊이 있게 이해하는 것은 기술 면접 및 실무 환경에서 독보적인 경쟁력을 제공한다.

특히 자율주행 분야의 네트워크 엔지니어는 "무선 채널의 비결정론적 특성을 어떻게 극복할 것인가?" 또는 "이종 네트워크 간의 정밀 시각 동기화를 어떻게 유지할 것인가?"와 같은 복합적인 문제에 대한 해결책을 제시할 수 있어야 한다. 네트워크 슬라이싱(Network Slicing), 소프트웨어 정의 네트워킹(SDN), 그리고 AI 기반의 자율 네트워크 운영 기술은 현대 IT 채용 시장에서 가장 선호되는 'Job-Ready' 기술들이다. 본 연구는 이러한 최첨단 기술들을 통합적으로 다룸으로써, 팀원들이 실제 산업 현장의 복잡한 네트워킹 과제를 해결할 수 있는 능력을 배양하도록 돕는다.

2. Problem Definition

2.1. 핵심 도전 과제: 비결정론적 환경에서의 결정론적 성능 보장

현재 자율주행 네트워크가 직면한 가장 큰 기술적 장벽은 무선 통신 환경의 불확실성 속에서 엄격한 결정론적(Deterministic) 성능을 제공해야 한다는 점이다. 전통적인 이더넷과 무선 LAN은 충돌 회피(CSMA/CA)나 통계적 다중화에 기반하여 '가급적 최선(Best-effort)'의 전송을 수행하지만, 이는 트래픽이 몰리는 상황에서 지연 시간이 무한정 늘어날 수 있는 구조적 한계를 지닌다. 자율주행 차량이 교차로에서 충돌 회피 명령을 수신할 때 이러한 불확실성은 치명적인 위험 요소가 된다.

또한 차량 내부의 전기/전자(E/E) 아키텍처가 영역 기반(Zone-based)으로 진화함에 따라, 하나의 고속 이더넷 백본 위에 안전 데이터, 센서 데이터, 인포테인먼트 트래픽이 혼재하게 되었다. 이때 낮은 우선순위의 대용량 데이터 전송이 중요한 제어 명령의 전송을 방해하지 않도록 보장하는 트래픽 셰이핑(Traffic Shaping) 및 프레임 선점(Frame Preemption) 기술의 정교한 구현이 요구된다.

2.2. 탐구 및 분석의 범위: 유무선 통합 및 L3 계층으로의 확장

본 연구는 자율주행용 LCN의 구현을 위해 다음과 같은 세부 기술 영역을 탐구 범위로 설정한다.

유선 TSN 메커니즘(L2): IEEE 802.1Qbv(시간 인식 셰이퍼), 802.1Qbu(프레임 선점), 802.1CB(복제 및 제거) 기능을 통해 차량 내 네트워크의 지연 시간을 최소화하고 신뢰성을 극대화하는 방안을 분석한다. 특히 2025년 발간된 IEEE 802.1DG 자동차용 TSN 프로파일의 최신 내용을 반영한다.

5G URLLC 및 무선 TSN 통합: 3GPP 표준에 정의된 무선 구간의 저지연 기술(미니 슬롯, 비경쟁 기반 업링크)과 유선 TSN 네트워크를 브릿징하는 DS-TT(Device-side TSN Translator) 및 NW-TT(Network-side TSN Translator)의 구조를 심층 탐구한다.

IETF DetNet(L3): 다중 홉(Multi-hop) 환경의 광역 네트워크에서 지연 시간 경계(Bounded Latency)를 보장하기 위한 아키텍처와 패킷 복제 및 제거 기능(PREOF)을 통한 신뢰성 향상 기법을 분석한다.

에지 지능화 및 자율 관리: MEC 아키텍처 상에서 AI를 활용한 실시간 트래픽 분석과 네트워크 자원 최적화(Network Slicing)가 자율주행 서비스 품질(QoS)에 미치는 영향을 고찰한다.

2.3. 최종 목표 성과: 기술 가이드라인 및 시뮬레이션 데이터 도출

본 팀은 본 보고서를 통해 다음과 같은 구체적인 결과물을 제시하고자 한다.

종단 간(E2E) 지연 시간 분석 모델: 유선 TSN과 무선 5G URLLC가 결합된 통합 아키텍처 상에서의 지연 시간 발생 요인을 수식화하고, 각 구간별 지연 예산(Latency Budget)을 산출한다.

네트워크 성능 검증 데이터: NS-3 또는 OMNeT++ 시뮬레이터를 활용하여 자율주행 시나리오(예: 군집 주행, 원격 운전) 하에서 LCN 기술 적용 전후의 성능(지연, 지터, 패킷 손실률)을 비교한 데이터를 도출한다.

산업 표준 기반 기술 가이드라인: 자율주행 레벨별 최적의 네트워크 프로토콜 설정값과 장애 대응(Fail-safe) 메커니즘에 대한 기술적 권고안을 제시한다.

3. Plan and Timeline

3.1. 연구 방법론: 다층적 분석 및 시뮬레이션 기반 검증

본 연구는 이론적 분석과 실무 사례 연구, 그리고 기술적 검증을 병행하는 다층적 방법론을 채택한다.

표준 명세 및 백서 정밀 분석: IEEE, IETF, 3GPP의 최신 표준 문서를 기반으로 기술적 작동 원리를 파악한다. 또한 Cisco, Ericsson, Huawei 등 주요 장비 벤더의 백서를 통해 실제 산업 현장의 베스트 프랙티스를 수집한다.

컴포넌트 기반 아키텍처 설계: 자율주행 차량, 기지국(gNB), 에지 서버(MEC), 코어 네트워크(5GC)로 이어지는 전체 네트워크 경로를 모듈별로 설계하고, 각 모듈 간의 상호 운용성을 분석한다.

네트워크 시뮬레이션 (NS-3): 자율주행 차량의 이동성(Mobility)과 트래픽 패턴을 모델링하여 실제 환경과 유사한 시뮬레이션을 수행한다. 특히 SUMO와 같은 교통 시뮬레이터와 NS-3를 연동하여 통신 환경이 차량 제어에 미치는 영향을 분석한다.

3.2. 3주간의 세부 마일스톤 및 실행 계획

| 주차 | 단계 | 주요 활동 내용 | 목표 결과물 |
| --- | --- | --- | --- |
| 1~2주차 | 기초 연구 및 요구사항 분석 | - 자율주행 레벨별 네트워크 KPI 정의 - IEEE 802.1 TSN 및 5G URLLC 표준 분석 - 관련 산업 백서 및 최신 논문 리뷰 | 기술 요구사항 정의서, 문헌 분석 요약본 |
| 3~5주차 | 시스템 설계 및 시뮬레이션 구축 | - 통합 LCN 아키텍처(유무선 브릿징) 설계 - NS-3 시뮬레이션 환경 구축 및 파라미터 설정 - 트래픽 시나리오(군집 주행 등) 정의 | 네트워크 아키텍처 도면, 시뮬레이션 코드 초안 |
| 6~8주차 | 실험 수행 및 최종 보고서 작성 | - 시뮬레이션 수행 및 데이터 수집 - 성능 결과 분석 및 시각화(그래프/표) - 결론 도출 및 팀 내 피어 리뷰 수행 | 최종 중간 보고서, 시뮬레이션 분석 리포트 |
| 9주차 | 최종 검토 및 영상 촬영 | - 전체 시뮬레이션 최종 검증 및 결과 데이터의 신뢰성 검토 - 프로젝트 핵심 결과 시연 및 기술 설명 영상 촬영 | 최종 결과 시연 영상, 최종 기말 보고서 |

4. Individual Roles

팀원 4명의 역할을 기술적 전문 분야와 프로젝트 관리 필요성에 따라 다음과 같이 분담하였다.

| 팀원 성명 (학번) | 주요 역할 | 세부 담당 업무 및 책임 |
| --- | --- | --- |
| [전남규(2020271319)] | 무선 프로토콜 및 URLLC 분석 | - 3GPP 5G/6G URLLC 물리 계층 기술 분석 - 무선 구간의 지연 시간 요인 분석 및 최적화 전략 수립 - 5G 네트워크 슬라이싱 기술 조사 및 적용 방안 연구 |
| [전예규(2025270607)] | 유선 TSN 및 표준 전문가 | - IEEE 802.1 TSN(802.1DG 포함) 표준 정밀 분석 - 차량 내부 이더넷 아키텍처 설계 및 트래픽 제어 로직 연구 - 유무선 동기화(gPTP) 연동 메커니즘 분석 |
| [조승찬(2022270620)] | L3 DetNet 및 에지 아키텍처 및 영상촬영 | - IETF DetNet(RFC 8655 등) 아키텍처 및 L3 결정론적 경로 연구 - MEC 기반 지연 최적화 아키텍처 설계 및 사례 연구 - 클라우드 연동 보안 및 신뢰성 프레임워크 구축 - 최종 시연 영상 촬영 |
| [조석호(2021270668)] | 시뮬레이션 및 기술 기록 | - NS-3 기반 자율주행 네트워크 시뮬레이션 수행 및 데이터 추출 - 기술 문서 작성 및 서식 관리, 타임라인 및 마일스톤 체크 - 프로젝트 기록물(회의록 등) 관리 및 최종 보고서 검수 |

단, 시뮬레이션은 모든 인원이 할 수 있도록 다같이 진행하는 것을 기본으로 한다.

5. 지연 시간 감수형 네트워크의 기술적 심층 고찰

5.1. 차량 내 네트워크 아키텍처의 패러다임 변화

전통적인 차량 네트워크는 CAN(Controller Area Network)이나 LIN(Local Interconnect Network)과 같은 저속 통신 방식에 의존해 왔다. 그러나 자율주행 레벨이 높아짐에 따라 센서 데이터의 양이 기하급수적으로 증가하면서, 차량 내부 네트워크는 고속 이더넷 기반의 TSN으로 급격히 전환되고 있다. 특히 영역 기반 아키텍처(Zone Architecture)는 차량의 물리적 위치에 따라 센서와 액추에이터를 그룹화하고, 이를 고속 이더넷 백본으로 연결하여 배선의 복잡성을 줄이고 데이터 처리 효율을 높인다.

이러한 환경에서 **IEEE 802.1Qbv(Time-Aware Shaper)**는 네트워크의 '교통경찰' 역할을 수행한다. 전체 통신 시간을 주기적인 슬롯으로 나누고, 중요한 제어 데이터만을 위한 전용 시간 창(Gate open)을 할당함으로써 다른 일반 데이터와의 충돌을 원천적으로 차단한다. 시뮬레이션 연구에 따르면, 이러한 결정론적 스케줄링을 통해 미션 크리티컬 트래픽의 최악의 경우 지연 시간(Worst-case Latency)을 수십 마이크로초 이내로 묶어둘 수 있다.

5.2. 5G URLLC와 TSN의 통합 메커니즘 분석

자율주행 차량이 외부 인프라와 협력하기 위해서는 차량 내부의 TSN 네트워크가 무선 5G 구간으로 확장되어야 한다. 3GPP는 5G 시스템을 하나의 '가상 TSN 브릿지'로 취급함으로써 이를 구현한다. 이 통합 과정에서 가장 중요한 요소는 다음과 같다.

정밀 시각 동기화 (Time Synchronization): 자율주행 차량과 지상 인프라는 동일한 시간 기준(Grandmaster Clock)을 공유해야 한다. 5G 시스템은 IEEE 802.1AS 프로토콜을 지원하여 무선 구간에서도 마이크로초 미만의 오차로 시간을 동기화하며, 이는 여러 대의 차량이 협력하여 주행하는 군집 주행(Platooning)에서 핵심적인 역할을 한다.

QoS 매핑: TSN 계층의 우선순위(Priority Code Point, PCP)를 5G 계층의 QoS 흐름(5QI)으로 정확하게 매핑하여, 무선 구간에서도 TSN과 동일한 수준의 우선순위 처리가 이루어지도록 보장한다.

5.3. IETF DetNet을 통한 광역 결정론적 네트워킹

자율주행 차량이 원격지의 관제 센터와 통신하거나, 여러 도시를 가로지르는 광역 네트워크를 통과할 때는 2계층의 TSN만으로는 한계가 있다. 이때 3계층(IP) 기반의 IETF DetNet 기술이 적용된다. DetNet은 IP 라우터들 사이에서 자원을 예약하고 경로를 고정함으로써, 다중 홉 구간에서도 지연 시간 변동(Jitter)을 최소화한다.

특히 **패킷 복제 및 제거 기능(PREOF)**은 신뢰성을 비약적으로 향상시킨다. 동일한 제어 패킷을 서로 다른 경로(Path A, Path B)로 전송하고, 수신 측에서 먼저 도착한 패킷을 채택하고 나중에 오는 중복 패킷을 제거하는 방식이다. 이 기술은 무선 구간의 일시적인 신호 약화나 네트워크 장비의 장애 발생 시에도 중단 없는 데이터 전송을 가능하게 하여 자율주행의 안전성을 보장한다.

| 기술 표준 | 주요 메커니즘 | 자율주행에서의 기대 효과 |
| --- | --- | --- |
| IEEE 802.1Qbv | 시간 인식 게이트 제어 (TAS) | 제어 명령의 전송 시간 확정성 확보 및 충돌 제거 |
| IEEE 802.1Qbu | 프레임 선점 및 중단 (Preemption) | 저우선순위 트래픽 전송 중 즉각적인 중요 데이터 삽입 |
| 3GPP URLLC | 미니 슬롯 및 비경쟁 접근 | 무선 구간 대기 시간 최소화 ( ms) |
| IETF DetNet | L3 자원 예약 및 PREOF | 광역 네트워크 및 이종 망 구간의 신뢰성/저지연 보장 |
| ETSI MEC | 컴퓨팅 자원 전진 배치 | 클라우드 처리 지연 제거 및 실시간 센서 데이터 분석 |

5.4. 미래 전망: 6G 및 AI 네이티브 자율 네트워크

다가오는 6G 시대의 자율주행 네트워크는 더욱 지능화될 것으로 예상된다. 네트워크 자체가 AI 모델의 추론 엔진으로 동작하여 차량의 센싱 데이터를 분석하고, 최적의 주행 경로를 가이드하는 '네트워크 보조 자율주행'이 현실화될 것이다. 또한 통신과 센싱의 통합(ISAC) 기술은 기지국이 레이더처럼 주변 차량과 보행자를 직접 감지하여 그 정보를 네트워크를 통해 차량에 전송함으로써, 차량 센서의 물리적 한계(사각지대 등)를 극복하게 할 것이다.

이러한 고도화된 시스템의 관리 및 운영을 위해 **인텐트 기반 네트워킹(Intent-based Networking)**이 도입되고 있다. 운영자가 "지연 시간 5ms 이하 유지"라는 상위 수준의 의도(Intent)만 입력하면, AI 에이전트가 네트워크 전체의 슬라이싱 파라미터와 라우팅 경로를 실시간으로 조정하여 해당 조건을 만족시키는 방식이다. 이는 자율주행이라는 복잡하고 역동적인 환경에서 인간의 개입 없이도 네트워크의 최적 성능을 유지하는 결정적인 기술이 될 것이다.

6. 결론

본 보고서는 자율주행 분야의 지연 시간 감수형 네트워크가 갖추어야 할 기술적 요건과 이를 구현하기 위한 유무선 통합 아키텍처를 상세히 고찰하였다. 자율주행의 안전성과 효율성을 보장하기 위해서는 단순히 개별 기술의 성능 향상을 넘어, 유선 TSN의 결정론적 제어와 무선 URLLC의 초고속 전송, 그리고 MEC 및 DetNet의 구조적 최적화가 유기적으로 결합되어야 함을 확인하였다.

제안된 9주간의 실행 계획을 통해, 우리 팀은 실제 산업 표준에 부합하는 네트워크 모델을 설계하고 시뮬레이션 기반의 정량적 데이터를 도출함으로써 자율주행 인프라 구축을 위한 실질적인 기술 가이드를 완성할 것이다. 이 과정에서 얻은 기술적 통찰과 실무 역량은 차세대 네트워크 전문가로서의 전문성을 증명하는 강력한 자산이 될 것으로 기대한다.

참고 자료

White Paper: Time-Sensitive Networking (TSN) and 5G Integration: Enabling Deterministic and Ultra-Reliable Communications - Benetel, 3월 26, 2026에 액세스, https://benetel.com/benetel-white-paper-time-sensitive-networking-tsn-and-5g-integration-enabling-deterministic-and-ultra-reliable-communications/

Understanding 5G URLLC: The Backbone of Modern Connectivity - Wray Castle, 3월 26, 2026에 액세스, https://wraycastle.com/blogs/knowledge-base/understanding-5g-urllc-the-backbone-of-modern-connectivity

A Comprehensive Survey of 5G URLLC and Challenges in the 6G Era - arXiv, 3월 26, 2026에 액세스, https://arxiv.org/html/2508.20205v1

5G NR Interview Questions & Answers | PDF | Computer Network - Scribd, 3월 26, 2026에 액세스, https://www.scribd.com/document/689706678/5G-question-and-answer

Striding Towards the Intelligent World White Paper 2024: Autonomous Driving Network (ADN) - Huawei, 3월 26, 2026에 액세스, https://www-file.huawei.com/-/media/corp2020/pdf/giv/striding-towards-the-intelligent-world/2024/intelligent_world_adn_2024_en.pdf?la=en

Understanding 5G & Time Critical Services, 3월 26, 2026에 액세스, https://www.5gamericas.org/understanding-5g-time-critical-services/

5G URLLC: Transforming Industries with Ultra-Reliable Low Latency Communications, 3월 26, 2026에 액세스, https://www.canogaperkins.net/post/5g-time-sensitive-networking-tsn-the-future-of-deterministic-communication-1

뉴스룸 | 현대모비스, 3월 26, 2026에 액세스, https://www.mobis.com/kr/aboutus/press.do?category=press&idx=5676

[K-TECH 글로벌 리더스] 〈현대차그룹②〉2026 자율주행 리포트… '제조 거인' 현대차, AI 소프트웨어로 판을 뒤집는다 - 동아일보, 3월 26, 2026에 액세스, https://www.donga.com/news/K-TECH%2BGlobal%2BLeaders/article/all/20260320/133569458/1

Ultra-Reliable Low-Latency Communications in Autonomous Vehicular Networks | Request PDF - ResearchGate, 3월 26, 2026에 액세스, https://www.researchgate.net/publication/331622784_Ultra-Reliable_Low-Latency_Communications_in_Autonomous_Vehicular_Networks

Automotive Ethernet Time-Sensitive Networking Market Research Report 2034 - Dataintelo, 3월 26, 2026에 액세스, https://dataintelo.com/report/automotive-ethernet-time-sensitive-networking-market

An Autonomous Vehicles Network with 5G URLLC Technology | Solution - Gigabyte, 3월 26, 2026에 액세스, https://www.gigabyte.com/Solutions/urllc

RFC 9450 - » RFC Editor, 3월 26, 2026에 액세스, https://www.rfc-editor.org/rfc/rfc9450.txt

Autonomous Vehicles: Reshaping IT Infrastructure - CoreSite, 3월 26, 2026에 액세스, https://www.coresite.com/blog/autonomous-vehicles-reshaping-it-infrastructure

Network Latency in Teleoperation of Connected and Autonomous Vehicles: A Review of Trends, Challenges, and Mitigation Strategies - PMC, 3월 26, 2026에 액세스, https://pmc.ncbi.nlm.nih.gov/articles/PMC11207977/

MEC vs. Edge Computing: Unraveling 5G Network Power - Penguin Solutions, 3월 26, 2026에 액세스, https://www.penguinsolutions.com/en-us/resources/blog/what-is-the-difference-between-mec-multi-access-edge-computing-and-edge-computing-network-technology-that-maximizes-the-transmission-capacity-of-5g-communication

Architecting operational models for private MEC using AWS Hybrid ..., 3월 26, 2026에 액세스, https://aws.amazon.com/blogs/industries/architecting-operational-models-for-private-mec-using-aws-hybrid-and-edge-services-with-verizon-private-5g-networks/

5 Latency Optimisation Tips for Private 5G - Firecell, 3월 26, 2026에 액세스, https://firecell.io/latency-optimisation-tips-for-private-5g/

5G and cloud technology for CIOs | Google Cloud Blog, 3월 26, 2026에 액세스, https://cloud.google.com/transform/5g-and-cloud-technology-for-cios

The AI Opportunity for Service Providers: Unlocking New Revenue Streams, Powered by Cisco Innovations White Paper, 3월 26, 2026에 액세스, https://www.cisco.com/c/en/us/solutions/collateral/artificial-intelligence/ai-opportunity-sp.html

Top asked 5G Technology Interview Questions | Vskills, 3월 26, 2026에 액세스, https://www.vskills.in/interview-questions/5g-technology-interview-questions

Comparative Performance Evaluation of 5G-TSN Applications in Indoor Factory Environments - arXiv, 3월 26, 2026에 액세스, https://arxiv.org/html/2501.12792v1

Time-Sensitive Networking (TSN) Test Solutions - Keysight, 3월 26, 2026에 액세스, https://www.keysight.com/us/en/learn/hubs/time-sensitive-networking.html

Interview Guide: Autonomous Driving Specialist - RoleCatcher, 3월 26, 2026에 액세스, https://rolecatcher.com/en/interviews/careers/professionals/science-and-engineering-professionals/engineering-professionals/engineering-professionals/autonomous-driving-specialist/

Entry Level Network Engineer Interview Questions - Resume Worded, 3월 26, 2026에 액세스, https://resumeworded.com/interview-questions/entry-level-network-engineer

Autonomous Vehicle Interview Questions - Complete Guide to Self-Driving Car Technology, 3월 26, 2026에 액세스, https://www.youtube.com/watch?v=e3hnY3Tn1UA

Adaptability: The Must-Have Skill for Network Engineers in the AI Era - Cisco Blogs, 3월 26, 2026에 액세스, https://blogs.cisco.com/learning/adaptability-the-must-have-skill-for-network-engineers-in-the-ai-era

AI in the evolution of Autonomous Networks - ETSI, 3월 26, 2026에 액세스, https://www.etsi.org/images/files/ETSIWhitePapers/ETSI-WP-69-AI-in_the_evolution_of_Autonomous_Networks.pdf

Challenges & Key Issues of Constructing 'MEC-Ready' 5G Bearer Networks for Carriers, 3월 26, 2026에 액세스, https://www.telecoms.com/5g-6g/challenges-key-issues-of-constructing-mec-ready-5g-bearer-networks-for-carriers

Top 30 Network Engineer Interview Questions And Answers- 591 Lab, 3월 26, 2026에 액세스, https://591lab.com/certification/network-engineer-interview-questions/

A Comprehensive Survey of Wireless Time-Sensitive Networking (TSN): Architecture, Technologies, Applications, and Open Issues - IEEE Xplore, 3월 26, 2026에 액세스, https://ieeexplore.ieee.org/iel8/9739/5451756/10735349.pdf

Time-Sensitive Network (TSN) Experiment in Sensor-Based Integrated Environment for Autonomous Driving - PMC, 3월 26, 2026에 액세스, https://pmc.ncbi.nlm.nih.gov/articles/PMC6427734/

Which Simulator to Choose for Next Generation Wireless Network Simulations? NS-3 or OMNeT++ - MDPI, 3월 26, 2026에 액세스, https://www.mdpi.com/2673-4591/46/1/36

Ultra-Low Latency (ULL) Networks: The IEEE TSN and IETF DetNet Standards and Related 5G ULL Research - SciSpace, 3월 26, 2026에 액세스, https://scispace.com/pdf/ultra-low-latency-ull-networks-the-ieee-tsn-and-ietf-detnet-lp6ztww5sf.pdf

5G-TSN Integration: The Future of Automotive Connectivity, 3월 26, 2026에 액세스, https://horizonconnect.de/5g-tsn-automotive-connectivity/

Level 5 by Layer 2: Time-Sensitive Networking for Autonomous Vehicles - ida.liu.se, 3월 26, 2026에 액세스, https://www.ida.liu.se/~sohsa65/courses/tsn-course-2021/selected%20papers/IEEE%20ComSoc%20-%20TSN%20special%20issue%202018/7%20-%20Level%205%20by%20Layer%202%20Time-Sensitive%20Networking%20for%20Autonomous%20Vehicles.pdf

TSN | - IEEE 802.1, 3월 26, 2026에 액세스, https://1.ieee802.org/tag/tsn/

Time-Sensitive Networking (TSN) Task Group | - IEEE 802.1, 3월 26, 2026에 액세스, https://1.ieee802.org/tsn/

Scalability Analysis of 5G-TSN Applications in Indoor Factory Settings - arXiv.org, 3월 26, 2026에 액세스, https://arxiv.org/html/2501.13138v1

(PDF) Deterministic Networking (DetNet) Security Considerations - ResearchGate, 3월 26, 2026에 액세스, https://www.researchgate.net/publication/353445745_Deterministic_Networking_DetNet_Security_Considerations

RFC 9566: Deterministic Networking (DetNet) Packet Replication, Elimination, and Ordering Functions (PREOF) via MPLS over UDP/IP, 3월 26, 2026에 액세스, https://www.rfc-editor.org/rfc/rfc9566.html

RFC 9633: Deterministic Networking (DetNet) YANG Data Model, 3월 26, 2026에 액세스, https://www.rfc-editor.org/rfc/rfc9633.pdf

Wireless-aware TSN Engineering: Implications for 5G and Upcoming 6G Networks - Deterministic6G, 3월 26, 2026에 액세스, https://deterministic6g.eu/images/publications/Wireless_aware_TSN_Engineering_Implications_for_5G_and_Upcoming_6G_Networks.pdf

doc: RFC 9320: Deterministic Networking (DetNet) Bounded Latency - hjp, 3월 26, 2026에 액세스, https://www.hjp.at/doc/rfc/rfc9320.html

Steps to Start Low Latency Communication Projects in NS3, 3월 26, 2026에 액세스, https://phdprojects.org/how-to-start-low-latency-communication-projects-using-ns3/

Network latency analysis in ns3, 3월 26, 2026에 액세스, https://ns3simulation.com/how-to-calculate-network-latency-in-ns3/

How AI-powered devices will drive the shift to uplink-heavy networks - Ericsson, 3월 26, 2026에 액세스, https://www.ericsson.com/en/blog/2026/3/ai-powered-devices-drive-the-shift-to-uplink-heavy-traffic

P802.1DG – TSN Profile for Automotive In-Vehicle Ethernet Communications | - IEEE 802, 3월 26, 2026에 액세스, https://1.ieee802.org/tsn/802-1dg/

usnistgov/ns3-cosim: Software to integrate ns-3 network models into co-simulations - GitHub, 3월 26, 2026에 액세스, https://github.com/usnistgov/ns3-cosim

Collaborative Research Framework for Automated Driving System Developers and Infrastructure Owners and Operators, 3월 26, 2026에 액세스, https://ops.fhwa.dot.gov/publications/fhwahop21012/fhwahop21012.pdf

Mind the gap: Developments in autonomous driving research and the sustainability challenge - PMC, 3월 26, 2026에 액세스, https://pmc.ncbi.nlm.nih.gov/articles/PMC7484706/

Network Latency in Teleoperation of Connected and Autonomous Vehicles: A Review of Trends, Challenges, and Mitigation Strategies - MDPI, 3월 26, 2026에 액세스, https://www.mdpi.com/1424-8220/24/12/3957

CN_Module5_MidpointReport_Format_Rubric.pdf

RFC 9450: Reliable and Available Wireless (RAW) Use Cases, 3월 26, 2026에 액세스, https://www.rfc-editor.org/rfc/rfc9450.html

Infrastructure Assisted Autonomous Driving: Research, Challenges, and Opportunities - IEEE Xplore, 3월 26, 2026에 액세스, https://ieeexplore.ieee.org/iel8/8782711/10774192/10887285.pdf

Autonomous Networks for Service Providers White Paper - Cisco, 3월 26, 2026에 액세스, https://www.cisco.com/c/en/us/solutions/collateral/networking/auton-ntwk-sp-wp.html

Entry Level Automotive Engineer Interview Questions & Answers - Teal, 3월 26, 2026에 액세스, https://www.tealhq.com/career-paths/entry-level-automotive-engineer-interview-questions/